#include<reg52.h>

#include<intrins.h>

#include "sim800.h"

#define uchar unsigned char

#define uint  unsigned int

#define K_MG_MV    120/66

typedef unsigned char  U8;       /* defined for unsigned 8-bits integer variable 	  无符号8位整型变量  */

typedef unsigned int   U16;      /* defined for unsigned 16-bits integer variable 	  无符号16位整型变量 */

/********测试GSM是否启动**********/

int test_boot;

/********测试GSM是否注册网络**********/

int test_net_register;

/********GSM串口接收数据缓存**********/

unsigned char idata GSM_receive[60];

/********GSM串口接收计数器**********/

unsigned char  GSMDATA_count;

unsigned char *content_temp =  "Warning: over temperature\r\n";

unsigned char *content_smoke = "Warning: excessive smoke concentration\r\n";

unsigned char *content_body =  "Warning: body entry\r\n";

unsigned char str_yw[12];//烟雾值

unsigned char T_baojing = 30;  //温度报警值

unsigned int  C_baojing = 500; //烟雾浓度报警值

unsigned char T_buff[3];  //报警缓存

unsigned char H_buff[3];

unsigned char C_buff[4];

bit  sf_flag=0; //设防标志

unsigned char moshi=0;

unsigned char data1;

long Value;

sbit bADcs=P3^4;

sbit bADcl=P3^6;

sbit bADda=P3^5;

sbit  sf_led = P1^3;//   设防指示灯

sbit  shefang= P1^4;//   设防按键

sbit  shezhi = P1^5; //  设置

sbit  jia    = P1^6; //	 加

sbit  jian   = P1^7; //  减

sbit  BUZZER = P2^0; //  LED蜂鸣器

sbit  Infra	 = P3^2; //  人体红外模块

unsigned char IntToString(unsigned char *str, int dat);

extern bit Start18B20();

extern bit Get18B20Temp(int *temp);				//	  ；；；；；、‘

extern void InitLcd1602();

extern void LcdShowStr(unsigned char x, unsigned char y, unsigned char *str);

float intT, decT;  //温度值的整数和小数部分

long kssj()   //开始收集

{

unsigned char i;

bADcs = 0;//当ADC0832未工作时其CS输入端应为高电平，此时芯片禁用.

bADcl=0;

bADda=1;

bADcl=1;

bADcl=0;//i down

bADda=1;

bADcl=1;

bADcl=0;	//   2 down

bADda=0;

bADcl=1;

bADcl=0;	//   3 down

bADda=1;

bADcl=1;

bADcl=0;	//   4 down

for(i=8;i>0;i--)

{

data1<<=1;

bADcl=0;

bADcl=1;

if(bADda==1) data1|=0x01;

bADda=1;

}

bADcs=1;

Value=data1*1.0/256*500;

Value=Value*K_MG_MV;

Value=Value-5;

if(Value<0)Value=0;

return Value;

}

void Key_set_scan()

{

if(shezhi==0)

{

Delay_Ms(10);

if(shezhi==0)

{

while(!shezhi);

InitLcd1602();     //初始化液晶

moshi++;

if(moshi >= 3)moshi = 0;

if(moshi == 0)

{

LcdShowStr(0, 0,"T:   C");

LcdShowStr(7, 0,"C:    PPM");

}

else if(moshi == 1)

{

LcdShowStr(0, 1,"Set_T:   C");

}

else if(moshi == 2)

{

LcdShowStr(0, 1,"Set_C:    PPM");

}

}

}

if(jia==0)

{

Delay_Ms(10);

if(jia==0)

{

while(!jia);

if(moshi==1)

{

T_baojing++ ;

if( T_baojing>=99 )T_baojing =99;

}

if(moshi==2)

{

C_baojing++ ;

if( C_baojing>=999 )C_baojing =999;

}

}

}

if(jian == 0)

{

Delay_Ms(10);

if(jian == 0)

{

while(!jian);

if(moshi==1)

{

T_baojing-- ;

if( T_baojing<=0 )T_baojing =0;

}

if(moshi==2)

{

C_baojing-- ;

if( C_baojing<=0 )C_baojing =0;

}

}

}

if(shefang == 0)

{

Delay_Ms(10);

if(shefang == 0)

{

while(!shefang);

sf_flag =~ sf_flag;

if(sf_flag == 1) //开启设防

sf_led = 0;

else 			 //关闭设防

{

sf_led = 1;

BUZZER = 1;

}

}

}

}

void wendu()

{

bit res;

int temp;  //读取到的当前温度值

unsigned char len;

unsigned char str_wd[5];   //温度缓冲区

Get18B20Temp(&temp);  //读取当前温度

res = Get18B20Temp(&temp);  //读取当前温度

if (res)                    //读取成功时，刷新当前温度显示

{

intT = temp >> 4;          //分离出温度值整数部分

decT = temp & 0xF;         //分离出温度值小数部分

len = IntToString(str_wd, intT); //整数部分转换为字符串

str_wd[len++] = '.';             //添加小数点

decT = (decT*10) / 16;        //二进制的小数部分转换为1位十进制位

str_wd[len++] = decT + '0';   //十进制小数位再转换为ASCII字符

str_wd[len] = '\0';              //添加字符串结束符

LcdShowStr(2, 0, str_wd);        //显示到液晶屏上

}

Start18B20();

}

/************主函数****************/

void main()

{

unsigned char i;

sf_flag = 0;

Uart_Init();

InitLcd1602();     //初始化液晶

Start18B20();

LcdShowStr(0,0,"  System init   ");

LcdShowStr(0,1,"  please wait   ");

for(i = 0;i < 15;i++)//等待网络稳定

{

Delay_Ms(1000);

}

InitLcd1602();

LcdShowStr(0, 0,"T:   C");

LcdShowStr(7, 0,"C:    PPM");

//		//测试是否已经开机，同时自适应波特率

//	while(test_boot==No)

//	{

//		Send_String("AT\r\n");

//		Delay_Ms(100);

//	}

//		test_boot=No;

//

//	//测试是否注册到网络

//	while(test_net_register==No)

//	{

//		Send_String("AT+CREG?\r\n");

//		Delay_Ms(100);

//	}

//	test_net_register=No;

Receive_message();      //配置成接受短信模式

while(1)

{

Key_set_scan();//按键扫描

if(moshi == 0)	  //设置模式切换

{

kssj();   //开始收集 ,电压采集检测烟雾浓度

Value =  Value-100;//烟雾浓度校准

IntToString(str_yw,Value); //转换成字符串

LcdShowStr(9, 0, str_yw); //烟雾浓度显示到液晶屏上

wendu();	 //温度检测并显示

if(sf_flag == 1) //当开启设防时

{

if(Value>=C_baojing||intT>T_baojing||Infra==1)   //有触发报警

{

BUZZER = 0;

Delay_Ms(1000);

LcdShowStr(0, 1,"                ");

if(intT > T_baojing)

{ 			  /*1234567890123456*/

LcdShowStr(0,1,"temp");

Send_message(content_temp);	  //发温度超标短信

}  else  LcdShowStr(0, 1,"    ");

if(Value >= C_baojing)

{ 				/*1234567890123456*/

LcdShowStr(5,1,"smoke");

Send_message(content_smoke);	  //发烟雾浓度超标短信

}  else  LcdShowStr(5, 1,"     ");

if(Infra == 1)

{ 			    /*1234567890123456*/

LcdShowStr(11, 1,"body");

Send_message(content_body);	  //发人体进入短信

}  else  LcdShowStr(11,1,"    ");

}

else 	//没有触发报警

{

BUZZER = 1;	 /*1234567890123456*/

LcdShowStr(0, 1," System normal  ");

}

}

}

else if(moshi == 1)	//设置温度上限模式

{

T_buff[0] = T_baojing/10+0x30;

T_buff[1] = T_baojing%10+0x30;

T_buff[2] = '\0';

LcdShowStr(6, 1,T_buff);

}

else if(moshi == 2)	 //设置烟雾浓度上限模式

{

C_buff[0] = C_baojing/100+0x30;

C_buff[1] = C_baojing%100/10+0x30;

C_buff[2] = C_baojing%10+0x30;

C_buff[4] = '\0';

LcdShowStr(6, 1,C_buff);

}

}

}

/* 整型数转换为字符串，str-字符串指针，dat-待转换数，返回值-字符串长度 */

unsigned char IntToString(unsigned char *str, int dat)

{

signed char i = 0;

unsigned char len = 0;

unsigned char buf[6];

if (dat < 0)  //如果为负数，首先

{

dat = -dat;

*str++ = '-';

len++;

}

do {          //先转换为低位在前的十进制数组

buf[i++] = dat % 10;

dat /= 10;

} while (dat > 0);

len += i;     //i最后的值就是有效字符的个数

while (i-- > 0)   //将数组值转换为ASCII码反向拷贝到接收指针上

{

*str++ = buf[i] + '0';

}

*str = '\0';  //添加字符串结束符

return len;   //返回字符串长度

}

/*****************************************************************************

串口中断

/****************************************************************************/

void uart(void) interrupt 4

{

unsigned char UART_data;

if(RI)

{

UART_data=SBUF;

if(UART_data=='\n')

{

if(GSM_receive[0]=='O'&&GSM_receive[1]=='K')

{

test_boot=Yes;

}

//else

//	test_boot=No;

if(GSM_receive[0]=='+'&&GSM_receive[1]=='C'&&GSM_receive[2]=='R'&&GSM_receive[3]=='E'&&GSM_receive[4]=='G')

{

if(GSM_receive[7]=='1'||GSM_receive[9]=='1'||GSM_receive[9]=='5')

{

test_net_register=Yes;

}

}

//	else

//  test_net_register=No;

if((GSM_receive[0]=='O')&&(GSM_receive[1]=='K'))	  //大写OK

{

// sf_flag = 0;

//sf_led = 1;

//BUZZER = 1;

}

if((GSM_receive[0]=='C')&&(GSM_receive[1]=='L')&&(GSM_receive[2]=='O')&&(GSM_receive[3]=='S')&&(GSM_receive[4]=='E'))

{

//  CZ = 1;LcdShowStr(5, 1,"Close");

}

GSMDATA_count=0;

}

else

{

GSM_receive[GSMDATA_count]=UART_data;

GSMDATA_count++;

}

}

RI=0;

}